ISSUED — FOR DEMONSTRATION ONLY

Contractor Safety & Environmental Plan (CSEP)

North Tower — SR-1042

Summit Ridge Constructors (self-performed steel package)

4100 Industrial Way, Austin, TX 78701

Status: Issued (demo) · Revision: R1 · 2026-04-27

# 1. Purpose and scope

This CSEP describes how Summit Ridge Constructors controls safety and health exposures during core-and-shell work at North Tower, including crane-assisted steel erection, temporary decking, leading-edge work, and street-side logistics. It satisfies pre-task planning, competent-person accountability, and coordination with the site PSHSEP.

# 2. Key roles

Project Manager: Avery Patel. Site safety lead: Maria Chen. Field supervision: Jordan Lee / Eli Brooks. Competent persons are designated per Subpart R steel erection and fall protection programs. All workers hold universal stop-work authority.

# 3. Site conditions and sequence

Active work includes Level 5 steel welding, crane picks, and material staging. Fire watch is mandatory for all hot work on temporary decking. Barricades and controlled access zones protect swing radius and leading edges. Night-shift MEP interfaces are coordinated through the GC logistics gate.

# 4. Hazard controls (summary)

## Fall protection

100% tie-off or equivalent approved systems at unprotected edges six feet and above. Controlled decking zones and access limits per erection plan. Rescue provisions identified before exposed work begins.

## Crane and rigging

Qualified signal person on all picks. No personnel under suspended loads. Lift plans reviewed when radius or capacity margins are tight.

## Hot work

Permit-required hot work with 60-minute fire watch minimum, non-combustible shields at deck penetrations, and documented handoff between shifts.

# 5. Emergency response

911 primary. Nearest ER communicated at site orientation. Assembly point: north parking lane. Spill kit and extinguisher maps posted at hoist landings.

# 6. Training and submittals

Toolbox talks logged daily for steel crew. MSHA/OSHA orientation cards on file. SDS for welding consumables and hydraulic fluids available at field office.

# 7. Document control

This issued CSEP is a static demo artifact for SafetyDocs360 walkthroughs. Live projects should replace placeholders with GC-approved versions and controlled revisions.